Part 1

Part 4

Part 5

Part 6

.define LED_ADDRESS 0x1000

.define SW_ADDRESS 0x3000

// Counter to count until 2^9 using leds on the DE1-SoC

// Can use SW to get delay.

mv    r6, #0                // Start counting from 0

MAIN:   mvt   r0, #LED_ADDRESS      // Point to LED reg address

add   r6, #1                // increment counter

st    r6, [r0]              // Store count to LED

mv    r3, #DELAY

ld    r3, [r3]

OUTER:  mvt   r1, #SW_ADDRESS       // Read SW address

ld    r4, [r1]              // load value from SW

add   r4, #1                // add 1 to SW value in case 0

INNER:  sub   r4, #1                // decrement outer loop

bne   #INNER                // If inner loop not 0, decrement again

sub   r3, #1                // decrement outer loop

bne   #OUTER                // If outer loop not 0, decrement again

b     #MAIN

DELAY: .word  8888

Part 7

.define HEX_ADDRESS 0x2000

.define SW_ADDRESS 0x3000

.define STACK 256				 // bottom of memory

======

// Count in Decimal from 0 to 65535 using the HEX displays

// r0 = counter register / remainder

// r1 = quotient

// r2 = digit register

// r3 = # of digits in number (up to 5)

// r4 = HEX_ADDRESS pointer

// r5 = return addresses / Seg7 digit value

// r6 = stack pointer

// pc = well, the pc

mv      r6, #STACK       // point to stack!

mv      r5, pc           // return address for subroutine

mv      pc, #BLANK       // Call subroutine

MAIN:   mv      r0, #0           // Initialize counter

LOOP:   mvt     r4, #HEX_ADDRESS // point to hex

mv      r3, #5           // 5 digits in 65535

sub     r6, #1           // store counter on stack

st      r0, [r6]

DISP:   mv      r5, pc           // return address

b       #DIV10           // divide by 10!

sub     r6, #1           // store r5

st      r5, [r6]

mv      r2, #DIGITS      // address of digit '0'

add     r2, r0           // get right digit

ld      r5, [r2]         // get actual digit!

st      r5, [r4]         // write into HEX0-HEX5

ld      r5, [r6]         // restore r5

add     r6, #1

mv      r0, r1           // put quotient in remainder

add     r4, #1           // putting into next HEX display

sub     r3, #1           // decrement # of digits left

bne     #DISP            // loop back for more digits

ld      r0, [r6]         // restore counter when done!!!

add     r6, #1

mv      r5, pc           // return address

b       #DELAYLOOP       // do the delay!

add     r0, #1           // counter r0 += 1

bcc     #LOOP            // continue until overflows!

mv      r5, pc           // return address for subroutine

mv      pc, #BLANK       // call subroutine to blank HEX!

b       #MAIN            // restart

DIV10:  sub     r6, #1           // Save modified registers

st      r2, [r6]         // Save on stack

mv      r1, #0           // Initialize Q

DLOOP:  mv      r2, #9           // check if r0 < 10

sub     r2, r0

bcc     #REDTIV          // if true, return

INC:    add     r1, #1           // if false, increment Q

sub     r0, #10          // r0 -= 10

b       #DLOOP           // loop again

REDTIV: ld      r2, [r6]         // restore stack

add     r6, #1

add     r5, #1           // adjust return address

mv      pc, r5           // return results!

// Subroutine BLANK

BLANK:  sub     r6, #1           // Save r0 into stack

st      r0, [r6]

sub     r6, #1           // Save r1 into stack

st      r0, [r6]

sub     r6, #1           // Save r2 into stack

st      r2, [r6]

mvt     r0, #HEX_ADDRESS // pointer to HEX0

mv      r1, #6           // Number of HEX displays

mv      r2, #0x0000      // Blank digit

LUP:    st      r2, [r0]         // Store blankness into HEX display

add     r0, #1           // Point to next HEX display!

sub     r1, #1           // decrement number of HEX displays left

bne     #LUP             // loop again if displays left

ld      r2, [r6]         // restore r2

add     r6, #1

ld      r1, [r6]         // restore r1

add     r6, #1

ld      r0, [r6]        //restore r0

add     r5, #1

mv      pc, r5           // Return back to where we were

// Delay subroutine

DELAYLOOP: sub  r6, #1           // store r1

st      r1, [r6]

sub     r6, #1           // store r3

st      r3, [r6]

sub     r6, #1           // store r4

st      r4, [r6]

mv      r3, #DELAY       // add the delay to r3

ld      r3, [r3]

OUTER:  mvt     r1, #SW_ADDRESS  // Read SW address

ld      r4, [r1]         // load value from SW

add     r4, #1           // add 1 to SW value in case 0

INNER:  sub     r4, #1           // decrement outer loop

bne     #INNER           // If inner loop not 0, decrement again

sub     r3, #1           // decrement outer loop

bne     #OUTER           // If outer loop not 0, decrement again

ld      r4, [r6]         // restore all registers

add     r6, #1

ld      r3, [r6]

add     r6, #1

ld      r1, [r6]

add     r6, #1

add     r5, #1

mv      pc, r5           // return back

// Data

DIGITS: .word 0b0000000000111111     // '0'

.word 0b0000000000000110     // '1'

.word 0b0000000001011011     // '2'

.word 0b0000000001001111     // '3'

.word 0b0000000001100110	 // '4'

.word 0b0000000001101101	 // '5'

.word 0b0000000001111101     // '6'

.word 0b0000000000000111     // '7'

.word 0b0000000001111111     // '8'

.word 0b0000000001101111     // '9'

DELAY:  .word  2222